Yes. This is a real production-grade Revit automation problem, not simply a “move the cable tray” script.

From your image, I understand the requirement as:

Detect a clash between a Cable Tray and Conduit, keep the conduit unchanged, and automatically reroute the Cable Tray around the Conduit while preserving the tray's width, height, type, level, orientation, connectivity and design intent.

The green result is essentially a 3D vertical offset / bridge route around the interfering conduit.

We should build this as a Cable Tray Clash Resolution Engine, not as a one-off script.

1. First: Define the Engineering Problem Correctly

Your red case is:

Original Cable Tray

───────────────────────────────

║

║ Conduit

║

───────────────────────────────

The required result is conceptually:

┌───────────────┐

/                 \

───────────────/                   \───────────────

↑                   ↑

rise up             come down

CONDUIT

│

│

In 3D, the tray should become something like:

P0 ───── P1

\

P2 ───────── P3

/

P4 ───── P5

Where:

- P0 = original tray start

- P1 = start of reroute

- P2 = raised tray

- P3 = raised tray

- P4 = return elevation

- P5 = original tray end

The actual number and orientation of segments will depend on the clash geometry.

2. The Most Important Senior-Level Decision

Do not start with Revit API coding.

The architecture should be:

┌──────────────────────┐

│   Revit Model        │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Clash Identification │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Geometry Extraction  │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Clash Analysis       │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Route Solver         │

│   Pure Geometry      │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Route Validation     │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Revit Model Writer   │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Final QA / Validation│

└──────────────────────┘

This separation is extremely important.

The geometry solver should not know that Revit exists.

3. Technical Knowledge Required

You asked specifically:

What technical knowledge should one have in Revit, Geometry and Coding?

I'd divide it into three major pillars.

A. REVIT KNOWLEDGE

You need strong knowledge of these Revit concepts.

A1. Revit MEP elements

You must understand:

MEPCurve

│

├── CableTray

├── Conduit

├── Duct

└── Pipe

Cable tray and conduit are not simply generic model geometry.

They have:

- connectors

- type

- dimensions

- level

- location curve

- MEP behavior

- fittings

- joins

- orientation

The Revit API exposes connectors on MEPCurve elements through ConnectorManager. Autodesk specifically documents that connectors on MEPCurve elements are created when the MEP curve is created. (Autodesk Help)

4. CableTray API

For this problem, you need to be comfortable with:

CableTray

CableTrayType

MEPCurve

LocationCurve

ConnectorManager

Connector

For example:

CableTray tray = ...;

LocationCurve location =

tray.Location as LocationCurve;

Curve curve = location.Curve;

XYZ start = curve.GetEndPoint(0);

XYZ end   = curve.GetEndPoint(1);

LocationCurve provides access to the curve defining a curve-based element, including getting and setting the curve. (Autodesk Help)

You also need:

tray.Width

tray.Height

tray.GetTypeId()

tray.ReferenceLevel

And importantly:

tray.ConnectorManager.Connectors

5. Cable Tray Creation

We will eventually create new tray segments using:

CableTray.Create(

doc,

trayTypeId,

startPoint,

endPoint,

levelId

);

The API supports creating Cable Tray from two points; this is also demonstrated in Autodesk-community API examples. (Autodesk Forums)

Then:

newTray.Width  = originalTray.Width;

newTray.Height = originalTray.Height;

This is important because the rerouted tray must retain the original tray properties.

6. Connectors Are Extremely Important

This problem cannot be solved reliably using only:

LocationCurve.Curve = ...

You need to understand:

CableTray

│

├── Start Connector

│

└── End Connector

and:

Connector.Origin

Connector.CoordinateSystem

Connector.Direction

Connector.Domain

Connector.IsConnected

Connector.AllRefs

Autodesk's API documentation confirms that connectors have an origin, direction/orientation information and connection relationships. (Autodesk Help)

This is what will allow us to build:

Tray Segment 1

│

▼

Connector

│

▼

Tray Segment 2

│

▼

Connector

│

▼

Tray Segment 3

7. Important Revit API Insight

One mistake I do not want us to make is blindly doing:

doc.Create.NewElbowFitting(...)

for Cable Tray.

NewElbowFitting is a generic MEP creation method, but its supported connector combinations need to be respected. Autodesk's MEP creation documentation lists the fitting creation APIs, while cable tray has its own electrical connector behavior. (Autodesk Help)

For this solution, our first implementation should use:

connector1.ConnectTo(connector2);

and let Revit resolve a required fitting where supported.

Autodesk explicitly documents that Connector.ConnectTo() can create a fitting between compatible connectors when necessary. (Autodesk Help)

This needs to be tested against the actual Cable Tray family/type configuration in your Revit 2024 environment.

8. Revit Geometry Knowledge

This is where the problem becomes interesting.

You need to understand:

Revit internal units

Revit internally works in feet.

Therefore:

100 mm

is not:

100

You need unit conversion.

For example:

double mm = 100;

double feet =

UnitUtils.ConvertToInternalUnits(

mm,

UnitTypeId.Millimeters);

This is essential for clearance calculations.

9. XYZ Mathematics

You need strong knowledge of:

XYZ

and vector operations:

Add()

Subtract()

Multiply()

DotProduct()

CrossProduct()

Normalize()

DistanceTo()

AngleTo()

For example:

XYZ trayDirection =

(trayEnd - trayStart).Normalize();

Then establish a local coordinate system.

10. Local Coordinate System — VERY IMPORTANT

This is probably the most important geometry concept in the entire project.

We should not assume:

Cable Tray = X direction

Conduit = Y direction

Real projects may contain:

0°

15°

30°

45°

90°

arbitrary rotated systems

Therefore we establish:

Tray Direction

│

▼

X'

Z'

↑

│

│

└──────── Y'

For a horizontal tray:

XYZ X = trayDirection.Normalize();

XYZ Z = XYZ.BasisZ;

XYZ Y = Z.CrossProduct(X).Normalize();

This gives us a tray-relative coordinate system.

Now we can say:

+X = along tray

+Y = across tray

+Z = vertical

That makes the algorithm much more robust.

11. Geometry Knowledge #2 — Solids

We need more than the location line.

A cable tray has physical dimensions:

Width

<------------>

┌────────┐

│        │ ↑

│        │ │ Height

└────────┘ ↓

A conduit has:

Diameter

<---->

◯

Therefore we need to work with solid geometry.

Typical API tools:

GeometryElement

GeometryInstance

Solid

Face

Edge

BoundingBoxXYZ

We need to obtain the physical solid of:

Cable Tray

+

Conduit

and determine their intersection.

12. Clash Detection

The first stage of our algorithm:

Cable Tray

│

▼

Get Solid

│

▼

Conduit

│

▼

Get Solid

│

▼

Boolean Intersection

│

▼

Intersection exists?

Conceptually:

Solid intersection =

BooleanOperationsUtils.ExecuteBooleanOperation(

traySolid,

conduitSolid,

BooleanOperationsType.Intersect);

If the resulting solid has meaningful volume:

CLASH = TRUE

Otherwise:

CLASH = FALSE

This is far better than simply checking bounding boxes.

13. Bounding Box vs Solid

We will use both.

Level 1 — Bounding box

Fast:

Tray BBox

↓

Conduit BBox

↓

Do they overlap?

If no:

STOP

If yes:

Potential clash

Level 2 — Solid intersection

Then:

Tray Solid

×

Conduit Solid

This determines actual interference.

This gives us a two-stage collision detection system:

Broad Phase

↓

Bounding Box

↓

Narrow Phase

↓

Solid Intersection

That's standard computational geometry thinking.

14. The Hardest Part: Finding the Reroute

Detecting the clash is actually not the hardest part.

The difficult part is:

Where should the new tray path go?

We need to calculate:

Original Tray Path

│

▼

Clash Region

│

▼

Required Clearance

│

▼

Candidate Routes

│

▼

Best Route

15. Clearance Calculation

Suppose:

Conduit diameter = 25 mm

Required clearance = 50 mm

Then the tray should not merely avoid the conduit.

It should maintain:

25/2 + 50 + trayHeight/2

depending on the actual routing direction.

More generally:

RequiredOffset =

ObstacleExtent

+

Clearance

+

TrayHalfThickness

For a vertical bypass, for example:

Conduit top

│

│

│  Clearance

▼

───────────────

Cable Tray

16. We Need a Clearance Envelope

This is a senior-level geometry technique.

Instead of thinking:

Conduit

we think:

Clearance Envelope

┌───────────────────────┐

│                       │

│       CONDUIT         │

│          │            │

│          │            │

└───────────────────────┘

The cable tray route must stay outside this envelope.

Conceptually:

Obstacle

+

Clearance

=

Forbidden Region

Then our routing algorithm simply finds a path around the forbidden region.

17. Candidate Route Generation

For your exact case, we can initially implement a deterministic vertical bypass solver.

For example:

P2────────────P3

/                \

/                  \

P0──────────P1                  P4──────────P5

CONDUIT

But we shouldn't hard-code only this.

The solver should generate candidates:

Candidate A

Go above:

┌──────────────┐

/                \

─────                    ─────

Candidate B

Go below:

─────                    ─────

\                /

└──────────────┘

Candidate C

Horizontal bypass.

Candidate D

If required, combined horizontal + vertical bypass.

Then score them.

18. Route Scoring

For each candidate:

Score =

routeLength

+ fittingPenalty

+ elevationPenalty

+ complexityPenalty

For example:

Candidate A

Length = 4200 mm

Fittings = 4

Elevation = 250 mm

Candidate B

Length = 4600 mm

Fittings = 4

Elevation = 250 mm

Candidate C

Length = 3900 mm

Fittings = 6

The solver might select:

Candidate A

because it is the shortest practical route.

This turns our script into a routing engine, rather than a macro.

19. But There Is Another Major Requirement

The route must satisfy constructability.

We cannot generate:

P1 ───── P2

│

│ 17 mm

│

P3

if the cable tray fitting cannot physically make that transition.

We need rules such as:

Minimum straight length

Minimum offset

Allowed bend angle

Preferred bend angle

Fitting availability

Clearance

For example:

45° preferred

90° allowed

30° allowed

These should be configurable.

20. Configuration, Not Hard Coding

We should create something like:

public class RoutingSettings

{

public double MinimumClearance { get; set; }

public double PreferredClearance { get; set; }

public double MinimumStraightLength { get; set; }

public double MaximumOffsetHeight { get; set; }

public bool AllowVerticalReroute { get; set; }

public bool AllowHorizontalReroute { get; set; }

public bool AllowBelowRoute { get; set; }

public bool AllowAboveRoute { get; set; }

}

This is much more professional.

21. The Architecture I Recommend

I would structure the project approximately like this:

CableTrayRouting/

│

├── Commands/

│   └── ResolveCableTrayClashCommand.cs

│

├── Models/

│   ├── ClashInfo.cs

│   ├── CableTrayInfo.cs

│   ├── ObstacleInfo.cs

│   ├── RouteCandidate.cs

│   └── RoutingSettings.cs

│

├── Revit/

│   ├── CableTrayService.cs

│   ├── ConnectorService.cs

│   ├── GeometryService.cs

│   ├── ElementService.cs

│   └── TransactionService.cs

│

├── Geometry/

│   ├── VectorUtils.cs

│   ├── CoordinateSystem.cs

│   ├── SolidUtils.cs

│   ├── IntersectionAnalyzer.cs

│   ├── ClearanceCalculator.cs

│   └── RouteSolver.cs

│

├── Routing/

│   ├── CandidateGenerator.cs

│   ├── RouteScorer.cs

│   ├── VerticalOffsetSolver.cs

│   └── HorizontalOffsetSolver.cs

│

├── Validation/

│   ├── ClashValidator.cs

│   ├── ClearanceValidator.cs

│   ├── ConnectivityValidator.cs

│   └── RouteValidator.cs

│

└── UI/

└── RoutingSettingsWindow.cs

That is the type of structure I would expect from a senior automation developer, rather than a 500-line external command.

22. The Complete Workflow

Our final command should eventually behave like this:

USER SELECTS CABLE TRAY

│

▼

SELECT / DETECT CONDUIT

│

▼

READ TRAY PROPERTIES

│

▼

EXTRACT GEOMETRY

│

▼

DETECT CLASH

│

├── NO CLASH → STOP

│

▼

ANALYZE CLASH

│

▼

CALCULATE CLEARANCE

│

▼

CREATE LOCAL COORDINATE SYSTEM

│

▼

GENERATE ROUTE CANDIDATES

│

▼

VALIDATE CANDIDATES

│

▼

SCORE CANDIDATES

│

▼

SELECT BEST ROUTE

│

▼

CREATE TRAY SEGMENTS

│

▼

CONNECT SEGMENTS

│

▼

CREATE / RESOLVE FITTINGS

│

▼

REGENERATE

│

▼

RECHECK CLASH

│

▼

CHECK CLEARANCE

│

▼

CHECK CONNECTIVITY

│

▼

SUCCESS

23. Very Important: We Don't Immediately Delete the Original Tray

A common beginner approach is:

Delete original

↓

Create new tray

I don't recommend that.

Instead:

Original Tray

│

├── Capture data

│

├── Capture connectors

│

├── Capture parameters

│

└── Capture relationships

│

▼

Create route

│

▼

Validate route

│

▼

Delete/replace

Even better, initially create the solution as preview geometry.

Only after validation:

COMMIT SOLUTION

24. Transaction Strategy

We should use:

TransactionGroup

│

├── Transaction

│      └── Create candidate

│

├── Regenerate

│

├── Validate

│

└── Rollback if failed

For multiple candidates:

Candidate A

↓

SubTransaction

↓

Validate

↓

Rollback

Candidate B

↓

SubTransaction

↓

Validate

↓

Rollback

Candidate C

↓

SubTransaction

↓

Validate

↓

Commit

This is exactly the sort of technique I want us to use.

We don't need to guess which route works.

We can actually test routes inside Revit.

25. The Algorithm I Recommend for Version 1

For your image, I would not start with a generic AI routing engine.

Start with:

VerticalOffsetCableTraySolver

Input:

CableTray

Conduit

Clearance

Output:

RoutePoints

Example:

List<XYZ> routePoints =

solver.Solve(tray, conduit, settings);

Result:

P0

P1

P2

P3

P4

P5

Then:

CableTrayCreator.CreateRoute(

doc,

tray,

routePoints);

26. Geometry Solver — Conceptual Implementation

The solver will perform:

Step 1

Get tray centerline:

T0 ---------------------------- T1

Step 2

Get conduit axis:

C0

│

│

│

C1

Step 3

Determine intersection/clash region.

Step 4

Transform everything into tray-local coordinates:

Z

↑

│

│

└────────→ X

Step 5

Calculate the forbidden region around the conduit.

Step 6

Calculate:

P1 = before clash

P2 = raised point

P3 = raised point

P4 = after clash

Step 7

Convert points back into global Revit coordinates.

27. One More Geometry Problem: Diagonal Conduits

The image appears to show the conduit crossing the tray at an angle.

Therefore:

Do not assume the conduit is perpendicular to the tray.

We need to calculate:

double angle =

trayDirection.AngleTo(conduitDirection);

And then classify:

0°        Parallel

0–30°     Shallow crossing

30–60°    Oblique crossing

60–90°    Near perpendicular

The route calculation should adapt accordingly.

28. What About Multiple Conduits?

Eventually we should support:

Cable Tray

│

├── Conduit A

├── Conduit B

├── Conduit C

└── Conduit D

Then:

┌─────────────┐

/               \

───────┐      /                 \──────

│─────                    │

│  C1       C2            │

│                          │

│       C3                 │

The solver should merge all obstacles:

Obstacle A

Obstacle B

Obstacle C

↓

Combined Forbidden Region

↓

One optimized route

That's where the system becomes genuinely valuable for a BIM production team.

29. Coding Knowledge You Need

For this particular project, I'd rate the required skills like this:

| Skill | Required Level |
| --- | --- |
| C# | ⭐⭐⭐⭐⭐ |
| Revit API | ⭐⭐⭐⭐⭐ |
| Revit MEP | ⭐⭐⭐⭐⭐ |
| XYZ/vector mathematics | ⭐⭐⭐⭐⭐ |
| Solid geometry | ⭐⭐⭐⭐⭐ |
| Transactions | ⭐⭐⭐⭐⭐ |
| Connectors | ⭐⭐⭐⭐⭐ |
| LINQ | ⭐⭐⭐⭐ |
| Debugging | ⭐⭐⭐⭐⭐ |
| Design patterns | ⭐⭐⭐⭐ |
| Dynamo | ⭐⭐⭐ |
| Python | ⭐⭐⭐ |
| Algorithms | ⭐⭐⭐⭐⭐ |
| Computational geometry | ⭐⭐⭐⭐⭐ |
| UI/WPF | ⭐⭐⭐ |
| Git | ⭐⭐⭐⭐ |

For this problem specifically, I would prioritize C# + Revit API + computational geometry over Dynamo.

30. What I Would NOT Do

I would avoid this type of implementation:

if (clash)

{

tray.Location.Move(

new XYZ(0, 0, 100 / 304.8));

}

That isn't a clash solver.

Also avoid:

if (conduit.Location.Curve.Direction.X > 0)

{

...

}

because it becomes useless when the model rotates.

And avoid:

hardcoded X

hardcoded Y

hardcoded Z

Instead:

Model coordinates

↓

Tray coordinate system

↓

Solve

↓

Transform back

31. The Engineering Stack

For this particular automation, I'd use:

Revit

Revit 2024

Development

Visual Studio

Language

C#

API

Revit API 2024

Geometry

Autodesk.Revit.DB

UI

Initially:

WPF

Later:

WPF + MVVM

Version control

Git

GitHub / company repository

Debugging

RevitLookup

Visual Studio debugger

Revit journal

custom logging

Revit's API is installed with Revit, and Autodesk provides the Revit API SDK as part of the Revit development environment. (Autodesk Help)

32. Our Development Roadmap for THIS Problem

I suggest we build it in 8 controlled stages.

Phase 01 — Revit Selection

Build:

Select Cable Tray

Select Conduit

Output:

Tray ID

Conduit ID

Phase 02 — Geometry Extraction

Build:

GetTrayGeometry()

GetConduitGeometry()

GetTrayDirection()

GetConduitDirection()

GetTrayDimensions()

GetConduitDiameter()

Phase 03 — Clash Engine

Build:

BoundingBoxClash()

SolidClash()

Output:

ClashResult

Phase 04 — Geometry Solver

Build:

CreateLocalCoordinateSystem()

CalculateClashRegion()

CalculateClearanceEnvelope()

GenerateCandidateRoutes()

This is the computational design core.

Phase 05 — Route Validation

Build:

ValidateClearance()

ValidateMinimumLength()

ValidateAngles()

ValidateSelfIntersection()

ValidateObstacleIntersection()

Phase 06 — Revit Route Creation

Build:

CreateCableTraySegment()

ConnectSegments()

CreateFittings()

CopyProperties()

Phase 07 — Post-Processing

Verify:

No clash

Correct clearance

Correct dimensions

Correct type

Correct elevation

Connected segments

No unintended elements

Phase 08 — Production UI

Finally:

┌──────────────────────────────────────┐

│ Cable Tray Clash Resolver            │

├──────────────────────────────────────┤

│                                      │

│ Clearance:        [ 50 mm ]          │

│                                      │

│ Preferred Offset: [ 250 mm ]         │

│                                      │

│ Route:             [ Above ▼ ]       │

│                                      │

│ ☑ Preserve Tray Type                 │

│ ☑ Preserve Width/Height              │

│ ☑ Auto Connect Fittings              │

│ ☑ Validate After Creation             │

│                                      │

│       [ Preview ] [ Resolve ]        │

└──────────────────────────────────────┘

33. And Here Is How I Want to Drive This With You

We should not jump straight into a huge 1,000-line code dump.

We'll build this like a professional software project.

Sprint 1

Revit element analysis

We'll write the first working C# command:

Select Cable Tray

Select Conduit

↓

Read:

Type

Width

Height

Level

Location

Direction

Connectors

Geometry

↓

Display diagnostic information

Sprint 2

Clash engine

Tray ↔ Conduit

Sprint 3

3D geometry solver

Sprint 4

Route candidate generation

Sprint 5

Create actual Cable Tray segments

Sprint 6

Fittings + connectors

Sprint 7

Validation

Sprint 8

Production UI + logging + error handling

34. The Senior-Level Goal

I don't want the final result to be:

“A script that works for the screenshot.”

I want the final system to be:

A reusable Revit Cable Tray Clash Resolution Engine that can solve the same class of problem for different tray orientations, conduit orientations, tray sizes, clearances and clash locations.

That distinction is very important for your Computational Design & BIM Services role.

The key technical concepts you need to master for this problem are therefore:

REVIT

│

├── CableTray / Conduit

├── MEPCurve

├── Connectors

├── Fittings

├── LocationCurve

├── Revit transactions

└── Revit geometry

GEOMETRY

│

├── XYZ vectors

├── Coordinate systems

├── Dot/Cross product

├── Projection

├── Line/Plane intersection

├── Bounding boxes

├── Solids

├── Boolean operations

├── Clearance envelopes

└── Path planning

CODING

│

├── C#

├── OOP

├── SOLID principles

├── LINQ

├── Algorithms

├── Design patterns

├── Transactions

├── Exception handling

├── Logging

└── Unit testing

COMPUTATIONAL DESIGN

│

├── Constraint solving

├── Candidate generation

├── Route optimization

├── Scoring

└── Validation

Autodesk's documentation also confirms that Cable Tray types expose properties such as bend multiplier and fitting-related information, which will become useful when we move from pure geometry into fitting-aware routing. (Autodesk Help)

🔥 Next implementation step

I recommend that we now start Sprint 1 immediately.

I will drive it with you as a senior developer and build the actual Revit 2024 C# solution, beginning with:

CableTrayClashResolverCommand.cs

The first version will:

- Select the Cable Tray.

- Select the Conduit.

- Extract their geometry.

- Extract tray width/height and conduit diameter.

- Extract their centerlines and directions.

- Extract connectors.

- Detect the actual solid clash.

- Calculate the clash region.

- Print a structured diagnostic report.

Then we will use that real data to build the green rerouting algorithm, rather than guessing geometry from the screenshot.

That is the correct engineering path for getting this to a complete, working production solution.

Yes — this correction changes the architecture in an important way.

We are not building a selection-based tool.

The tool will operate like:

ACTIVE REVIT DOCUMENT

│

├── Collect ALL Cable Trays

│

├── Collect ALL Conduits

│

▼

Broad-phase clash check

│

▼

Solid-vs-Solid clash check

│

▼

Identify clash region

│

▼

Calculate reroute

│

▼

Create new Cable Tray path

│

▼

Validate

And your second image makes the intended routing much clearer:

BLACK  = existing Cable Tray

GREEN  = Conduit

PINK   = automatically generated Cable Tray bypass

The pink solution is essentially:

existing elevation

│

│  diagonal rise

▼

────────────────

elevated tray

│

│ diagonal drop

▼

existing elevation

So let's build V0.1 as a real Revit 2024 C# add-in.

1. What this first version will do

This test version will:

Automatically

- collect every CableTray

- collect every Conduit

- compare their bounding boxes

- perform actual solid intersection

- identify cable-tray/conduit clashes

- determine where along the tray the clash occurs

- calculate an elevated bypass

- create actual Cable Tray elements

- preserve:

  - Cable Tray type

  - width

  - height

  - reference level

- create the route in this pattern:

P0 ───── P1

\

P2 ───────── P3

\

P4 ───── P5

- color the newly created tray pink

- NOT delete or modify the original tray

That last point is deliberate. We want to test the geometry first before allowing the automation to modify production model elements.

Revit's API supports creating Cable Tray directly from start/end points, and Revit's Boolean geometry API supports solid intersection operations. (Revit API Docs)

2. Important scope of V0.1

I want to be very clear about this.

This is our first working geometry engine, not yet the final production solver.

V0.1 assumptions

It handles:

- native Revit Cable Tray

- native Revit Conduit

- active project document

- straight Cable Tray

- approximately horizontal Cable Tray

- automatic collection

- one clash per original tray

- reroute above the conduit

- no manual selection

- original tray remains untouched

It intentionally does not yet solve:

- multiple clashes on the same tray

- linked models

- existing tray fittings

- existing connected systems

- optimum above/below selection

- minimum bend radius optimization

- automatic deletion/splitting of original tray

- automatic fitting creation

- complex sloped trays

- multiple obstacles forming a combined forbidden region

Those are Phase 2+, not something I want to hide inside a first prototype.

3. The actual V0.1 C# code

Create a Revit 2024 Class Library and add this class:

ResolveCableTrayClashes.cs

using System;

using System.Collections.Generic;

using System.Linq;

using Autodesk.Revit.Attributes;

using Autodesk.Revit.DB;

using Autodesk.Revit.DB.Electrical;

using Autodesk.Revit.UI;

namespace CableTrayClashResolver

{

[Transaction(TransactionMode.Manual)]

public class ResolveCableTrayClashes : IExternalCommand

{

// ------------------------------------------------------------

// UNIT CONVERSION

// Revit internal length unit = feet

// ------------------------------------------------------------

private const double MM = 1.0 / 304.8;

// ------------------------------------------------------------

// TEST SETTINGS

// ------------------------------------------------------------

// Required clearance between conduit and new tray.

private const double ClearanceMm = 50.0;

// Minimum vertical rise.

private const double MinimumRiseMm = 100.0;

// Minimum horizontal transition distance.

private const double MinimumTransitionMm = 300.0;

// Revit geometry tolerance.

private const double GeometryTolerance = 1e-9;

// ============================================================

// MAIN COMMAND

// ============================================================

public Result Execute(

ExternalCommandData commandData,

ref string message,

ElementSet elements)

{

UIDocument uidoc =

commandData.Application.ActiveUIDocument;

Document doc = uidoc.Document;

try

{

// ----------------------------------------------------

// 1. COLLECT ALL CABLE TRAYS

// ----------------------------------------------------

List<CableTray> trays =

new FilteredElementCollector(doc)

.OfClass(typeof(CableTray))

.WhereElementIsNotElementType()

.Cast<CableTray>()

.ToList();

// ----------------------------------------------------

// 2. COLLECT ALL CONDUITS

// ----------------------------------------------------

List<Conduit> conduits =

new FilteredElementCollector(doc)

.OfClass(typeof(Conduit))

.WhereElementIsNotElementType()

.Cast<Conduit>()

.ToList();

if (trays.Count == 0 || conduits.Count == 0)

{

TaskDialog.Show(

"Cable Tray Clash Resolver",

$"Cable Trays found : {trays.Count}\n" +

$"Conduits found    : {conduits.Count}\n\n" +

"Nothing to process."

);

return Result.Succeeded;

}

// ----------------------------------------------------

// 3. FIND REAL CLASHES

// ----------------------------------------------------

List<ClashInfo> clashes =

FindClashes(

trays,

conduits);

if (clashes.Count == 0)

{

TaskDialog.Show(

"Cable Tray Clash Resolver",

$"Cable Trays found : {trays.Count}\n" +

$"Conduits found    : {conduits.Count}\n\n" +

"No solid clashes detected."

);

return Result.Succeeded;

}

// ----------------------------------------------------

// 4. CREATE REROUTES

// ----------------------------------------------------

int createdSegments = 0;

int skipped = 0;

HashSet<long> processedTrays =

new HashSet<long>();

using (Transaction tx =

new Transaction(

doc,

"Cable Tray Clash Test Reroute"))

{

tx.Start();

foreach (ClashInfo clash in clashes)

{

// ------------------------------------------------

// V0.1:

// Only one clash is processed per original tray.

// ------------------------------------------------

if (processedTrays.Contains(

clash.Tray.Id.Value))

{

skipped++;

continue;

}

// ------------------------------------------------

// BUILD ROUTE

// ------------------------------------------------

List<XYZ> routePoints;

if (!TryBuildVerticalBypass(

clash,

out routePoints))

{

skipped++;

continue;

}

// ------------------------------------------------

// CREATE ACTUAL REVIT CABLE TRAYS

// ------------------------------------------------

List<CableTray> created =

CreateRoute(

doc,

clash.Tray,

routePoints);

// ------------------------------------------------

// COLOR NEW ROUTE PINK

// ------------------------------------------------

foreach (CableTray newTray in created)

{

OverrideAsPink(

doc.ActiveView,

newTray.Id);

}

createdSegments += created.Count;

processedTrays.Add(

clash.Tray.Id.Value);

}

tx.Commit();

}

// ----------------------------------------------------

// RESULT

// ----------------------------------------------------

TaskDialog.Show(

"Cable Tray Clash Resolver",

$"Cable Trays found : {trays.Count}\n" +

$"Conduits found    : {conduits.Count}\n" +

$"Clashes found     : {clashes.Count}\n" +

$"Reroutes created  : {processedTrays.Count}\n" +

$"New tray segments : {createdSegments}\n" +

$"Skipped            : {skipped}\n\n" +

"TEST MODE:\n" +

"Original Cable Trays were NOT modified or deleted."

);

return Result.Succeeded;

}

catch (Exception ex)

{

message = ex.ToString();

TaskDialog.Show(

"Cable Tray Clash Resolver - Error",

ex.ToString());

return Result.Failed;

}

}

// ============================================================

// CLASH DETECTION

// ============================================================

private static List<ClashInfo> FindClashes(

List<CableTray> trays,

List<Conduit> conduits)

{

List<ClashInfo> result =

new List<ClashInfo>();

// --------------------------------------------------------

// CACHE BOUNDING BOXES

// --------------------------------------------------------

Dictionary<long, BoundingBoxXYZ> trayBoxes =

trays.ToDictionary(

x => x.Id.Value,

x => x.get_BoundingBox(null));

Dictionary<long, BoundingBoxXYZ> conduitBoxes =

conduits.ToDictionary(

x => x.Id.Value,

x => x.get_BoundingBox(null));

// --------------------------------------------------------

// BROAD PHASE

// --------------------------------------------------------

foreach (CableTray tray in trays)

{

BoundingBoxXYZ trayBox =

trayBoxes[tray.Id.Value];

if (trayBox == null)

continue;

foreach (Conduit conduit in conduits)

{

BoundingBoxXYZ conduitBox =

conduitBoxes[conduit.Id.Value];

if (conduitBox == null)

continue;

// Fast bounding-box test.

if (!BoxesIntersect(

trayBox,

conduitBox,

1.0 * MM))

{

continue;

}

// ------------------------------------------------

// NARROW PHASE

// Actual Solid vs Solid

// ------------------------------------------------

List<Solid> traySolids =

GetSolids(tray);

List<Solid> conduitSolids =

GetSolids(conduit);

Solid intersection = null;

foreach (Solid traySolid in traySolids)

{

foreach (Solid conduitSolid in conduitSolids)

{

try

{

Solid test =

BooleanOperationsUtils

.ExecuteBooleanOperation(

traySolid,

conduitSolid,

BooleanOperationsType.Intersect);

if (test != null &&

test.Volume >

GeometryTolerance)

{

intersection = test;

break;

}

}

catch

{

// ------------------------------------------------

// Some complicated Revit geometry can cause

// boolean failures.

// Ignore that pair and continue.

// ------------------------------------------------

}

}

if (intersection != null)

break;

}

// ------------------------------------------------

// REAL CLASH FOUND

// ------------------------------------------------

if (intersection != null)

{

result.Add(

new ClashInfo

{

Tray = tray,

Conduit = conduit,

Intersection = intersection

});

}

}

}

return result;

}

// ============================================================

// BUILD REROUTE

// ============================================================

private static bool TryBuildVerticalBypass(

ClashInfo clash,

out List<XYZ> points)

{

points =

new List<XYZ>();

// --------------------------------------------------------

// GET CABLE TRAY LOCATION

// --------------------------------------------------------

LocationCurve lc =

clash.Tray.Location as LocationCurve;

if (lc == null)

return false;

if (!(lc.Curve is Line))

return false;

Line trayLine =

lc.Curve as Line;

XYZ start =

trayLine.GetEndPoint(0);

XYZ end =

trayLine.GetEndPoint(1);

XYZ direction =

(end - start).Normalize();

// --------------------------------------------------------

// V0.1 SUPPORTS HORIZONTAL STRAIGHT TRAYS

// --------------------------------------------------------

if (Math.Abs(direction.Z) > 0.01)

{

return false;

}

double trayLength =

start.DistanceTo(end);

// --------------------------------------------------------

// GET TRAY SIZE

// --------------------------------------------------------

double trayWidth =

GetParameterValue(

clash.Tray,

BuiltInParameter

.RBS_CABLETRAY_WIDTH_PARAM);

double trayHeight =

GetParameterValue(

clash.Tray,

BuiltInParameter

.RBS_CABLETRAY_HEIGHT_PARAM);

// --------------------------------------------------------

// GET CONDUIT BOUNDING BOX

// --------------------------------------------------------

BoundingBoxXYZ conduitBox =

clash.Conduit

.get_BoundingBox(null);

if (conduitBox == null)

return false;

// --------------------------------------------------------

// ORIGINAL TRAY ELEVATION

// --------------------------------------------------------

double currentZ =

start.Z;

// --------------------------------------------------------

// REQUIRED NEW ELEVATION

//

// conduit top

// + clearance

// + half tray height

//

// --------------------------------------------------------

double targetZ =

conduitBox.Max.Z

+ ClearanceMm * MM

+ trayHeight / 2.0;

// Make sure we actually rise.

targetZ =

Math.Max(

targetZ,

currentZ

+ MinimumRiseMm * MM);

double rise =

targetZ - currentZ;

// --------------------------------------------------------

// FIND CLASH EXTENT ALONG TRAY

// --------------------------------------------------------

double minStation;

double maxStation;

if (!TryGetStationRange(

clash.Intersection,

start,

direction,

out minStation,

out maxStation))

{

return false;

}

// --------------------------------------------------------

// TRANSITION LENGTH

//

// Larger of:

//

// vertical rise

// OR

// minimum transition

//

// This gives a reasonable diagonal transition.

// --------------------------------------------------------

double transition =

Math.Max(

rise,

MinimumTransitionMm * MM);

// --------------------------------------------------------

// ENTRY / EXIT STATIONS

// --------------------------------------------------------

double entryStation =

minStation - transition;

double exitStation =

maxStation + transition;

// Keep inside original tray.

entryStation =

Math.Max(

0,

entryStation);

exitStation =

Math.Min(

trayLength,

exitStation);

// --------------------------------------------------------

// VERIFY THERE IS ENOUGH TRAY

// --------------------------------------------------------

if (entryStation <= 1.0 * MM)

return false;

if (exitStation >=

trayLength - 1.0 * MM)

return false;

if (exitStation <= entryStation)

return false;

// --------------------------------------------------------

// ROUTE POINTS

//

// P0 ----- P1

//          \

//           P2 -------- P3

//                              \

//                               P4 ----- P5

//

// --------------------------------------------------------

XYZ p0 =

start;

XYZ p1 =

start

+ direction * entryStation;

XYZ p2 =

new XYZ(

p1.X,

p1.Y,

targetZ);

XYZ p3Base =

start

+ direction * exitStation;

XYZ p3 =

new XYZ(

p3Base.X,

p3Base.Y,

targetZ);

XYZ p4 =

p3Base;

XYZ p5 =

end;

// --------------------------------------------------------

// ADD POINTS

// --------------------------------------------------------

AddPoint(points, p0);

AddPoint(points, p1);

AddPoint(points, p2);

AddPoint(points, p3);

AddPoint(points, p4);

AddPoint(points, p5);

return points.Count >= 2;

}

// ============================================================

// CREATE CABLE TRAY SEGMENTS

// ============================================================

private static List<CableTray> CreateRoute(

Document doc,

CableTray originalTray,

List<XYZ> points)

{

List<CableTray> result =

new List<CableTray>();

ElementId typeId =

originalTray.GetTypeId();

ElementId levelId =

originalTray.ReferenceLevel.Id;

// --------------------------------------------------------

// ORIGINAL DIMENSIONS

// --------------------------------------------------------

double width =

GetParameterValue(

originalTray,

BuiltInParameter

.RBS_CABLETRAY_WIDTH_PARAM);

double height =

GetParameterValue(

originalTray,

BuiltInParameter

.RBS_CABLETRAY_HEIGHT_PARAM);

// --------------------------------------------------------

// CREATE EACH ROUTE SEGMENT

// --------------------------------------------------------

for (int i = 0;

i < points.Count - 1;

i++)

{

XYZ a =

points[i];

XYZ b =

points[i + 1];

// Ignore zero-length segments.

if (a.DistanceTo(b) <

1.0 * MM)

{

continue;

}

CableTray tray =

CableTray.Create(

doc,

typeId,

a,

b,

levelId);

// ----------------------------------------------------

// RESTORE ORIGINAL WIDTH / HEIGHT

// ----------------------------------------------------

Parameter widthParam =

tray.get_Parameter(

BuiltInParameter

.RBS_CABLETRAY_WIDTH_PARAM);

Parameter heightParam =

tray.get_Parameter(

BuiltInParameter

.RBS_CABLETRAY_HEIGHT_PARAM);

if (widthParam != null &&

!widthParam.IsReadOnly)

{

widthParam.Set(width);

}

if (heightParam != null &&

!heightParam.IsReadOnly)

{

heightParam.Set(height);

}

result.Add(tray);

}

return result;

}

// ============================================================

// PINK GRAPHICS

// ============================================================

private static void OverrideAsPink(

View view,

ElementId elementId)

{

OverrideGraphicSettings ogs =

new OverrideGraphicSettings();

ogs.SetProjectionLineColor(

new Color(

255,

0,

180));

ogs.SetProjectionLineWeight(6);

view.SetElementOverrides(

elementId,

ogs);

}

// ============================================================

// FIND CLASH EXTENT ALONG TRAY

// ============================================================

private static bool TryGetStationRange(

Solid solid,

XYZ origin,

XYZ direction,

out double minStation,

out double maxStation)

{

minStation =

double.MaxValue;

maxStation =

double.MinValue;

foreach (Edge edge in solid.Edges)

{

IList<XYZ> tessellated =

edge.Tessellate();

foreach (XYZ p in tessellated)

{

// Project intersection point onto

// cable tray direction.

double station =

(p - origin)

.DotProduct(direction);

minStation =

Math.Min(

minStation,

station);

maxStation =

Math.Max(

maxStation,

station);

}

}

return

minStation != double.MaxValue &&

maxStation != double.MinValue;

}

// ============================================================

// EXTRACT SOLIDS

// ============================================================

private static List<Solid> GetSolids(

Element element)

{

List<Solid> solids =

new List<Solid>();

Options options =

new Options

{

ComputeReferences = false,

DetailLevel = ViewDetailLevel.Fine,

IncludeNonVisibleObjects = false

};

GeometryElement geometry =

element.get_Geometry(options);

if (geometry == null)

return solids;

CollectSolids(

geometry,

solids);

return solids;

}

private static void CollectSolids(

GeometryElement geometry,

List<Solid> solids)

{

foreach (GeometryObject obj in geometry)

{

// -----------------------------------------------

// DIRECT SOLID

// -----------------------------------------------

Solid solid =

obj as Solid;

if (solid != null &&

solid.Volume >

GeometryTolerance)

{

solids.Add(solid);

continue;

}

// -----------------------------------------------

// GEOMETRY INSTANCE

// -----------------------------------------------

GeometryInstance instance =

obj as GeometryInstance;

if (instance != null)

{

GeometryElement

instanceGeometry =

instance.GetInstanceGeometry();

if (instanceGeometry != null)

{

CollectSolids(

instanceGeometry,

solids);

}

}

}

}

// ============================================================

// BOUNDING BOX INTERSECTION

// ============================================================

private static bool BoxesIntersect(

BoundingBoxXYZ a,

BoundingBoxXYZ b,

double tolerance)

{

return

a.Min.X <= b.Max.X + tolerance &&

a.Max.X + tolerance >= b.Min.X &&

a.Min.Y <= b.Max.Y + tolerance &&

a.Max.Y + tolerance >= b.Min.Y &&

a.Min.Z <= b.Max.Z + tolerance &&

a.Max.Z + tolerance >= b.Min.Z;

}

// ============================================================

// READ PARAMETER

// ============================================================

private static double GetParameterValue(

Element element,

BuiltInParameter parameter)

{

Parameter p =

element.get_Parameter(parameter);

if (p == null ||

!p.HasValue)

{

return 0.0;

}

return p.AsDouble();

}

// ============================================================

// ADD POINT WITHOUT DUPLICATES

// ============================================================

private static void AddPoint(

List<XYZ> points,

XYZ point)

{

if (points.Count == 0 ||

points[

points.Count - 1]

.DistanceTo(point)

> 1.0 * MM)

{

points.Add(point);

}

}

// ============================================================

// CLASH DATA

// ============================================================

private class ClashInfo

{

public CableTray Tray { get; set; }

public Conduit Conduit { get; set; }

public Solid Intersection { get; set; }

}

}

}

4. What this code actually does

Suppose your model contains:

Cable Tray 01

Cable Tray 02

Cable Tray 03

Cable Tray 04

Conduit 01

Conduit 02

Conduit 03

Conduit 04

You press the command.

There is no selection prompt.

The algorithm internally performs:

Cable Tray 01

│

├── Conduit 01

├── Conduit 02

├── Conduit 03

└── Conduit 04

Cable Tray 02

│

├── Conduit 01

├── Conduit 02

└── ...

But we don't immediately perform expensive solid Boolean operations on every pair.

We first do:

Bounding Box

↓

Possible clash?

↓

YES

↓

Solid intersection

That broad-phase/narrow-phase approach is important because bounding boxes can produce false positives for rotated geometry; Autodesk recommends using actual geometry when the bounding box doesn't closely represent the element's shape. (Autodesk Help)

5. How the actual reroute is calculated

Suppose the tray centerline is:

P0────────────────────────────────────────P5

The solid clash gives us a region:

CLASH

↓

P0────────────████──────────────P5

The algorithm projects the intersection geometry onto the tray's direction.

So we calculate:

minStation

maxStation

Then:

entry = minStation - transition

exit  = maxStation + transition

The route becomes:

P0 ───── P1

\

P2────────────────P3

\

P4 ───── P5

Exactly the principle from your pink example.

6. How the height is calculated

We don't hard-code:

Z + 500 mm

Instead:

New Tray Bottom

↑

│ Clearance

│

Conduit Top

The centerline elevation is calculated approximately as:

Target Tray Center Z

=

Conduit Top

+

Required Clearance

+

Half Tray Height

So if:

Conduit top = 2800 mm

Clearance   = 50 mm

Tray height = 100 mm

then:

Tray center

=

2800 + 50 + 50

= 2900 mm

This is already much closer to an engineering algorithm than simply moving the tray by an arbitrary amount.

7. Why I used the actual solid intersection

The clash engine is doing:

Cable Tray Solid

×

Conduit Solid

↓

Intersection Solid

rather than:

centerline × centerline

This is important because the centerlines can technically miss while the physical tray and conduit still interfere.

Revit's BooleanOperationsUtils.ExecuteBooleanOperation() is designed to perform Boolean operations on Revit solids and returns the resulting solid; its intersection volume can then be inspected. (Autodesk Help)

8. Why I'm not deleting the original tray yet

After running the command you'll intentionally see something similar to:

BEFORE

BLACK

──────────────────────────────

GREEN

╲

╲

and then:

AFTER TEST

BLACK

──────────────────────────────

GREEN

╲

╲

PINK

╱────────────╲

─────────╱              ╲─────────

This is exactly what I want for the first test.

We need to prove:

Geometry engine

works.

Clash engine

works.

Elevation calculation

works.

Revit element creation

works.

Only then should we make the tool destructive.

9. Revit 2024 project setup

Create:

Visual Studio

↓

Create New Project

↓

Class Library (.NET Framework)

↓

.NET Framework 4.8

Project name:

CableTrayClashResolver

Add references to:

RevitAPI.dll

RevitAPIUI.dll

from your Revit 2024 installation folder.

For example:

C:\Program Files\Autodesk\Revit 2024\

Build against the Revit 2024 API, not another Revit version. Revit 2024 also introduced the 64-bit ElementId.Value API, which is why the sample deliberately uses Id.Value instead of the old IntegerValue. (Autodesk Help)

10. Add the .addin file

Create:

CableTrayClashResolver.addin

inside:

%AppData%\Autodesk\Revit\Addins\2024\

Use:

<?xml version="1.0" encoding="utf-8"?>

<RevitAddIns>

<AddIn Type="Command">

<Name>

Cable Tray Clash Resolver

</Name>

<Assembly>

C:\YourPath\CableTrayClashResolver.dll

</Assembly>

<AddInId>

7A1B2C3D-4E5F-6789-ABCD-123456789001

</AddInId>

<FullClassName>

CableTrayClashResolver.ResolveCableTrayClashes

</FullClassName>

<VendorId>

CADAR

</VendorId>

<VendorDescription>

Cable Tray Automatic Clash Resolution

</VendorDescription>

</AddIn>

</RevitAddIns>

Replace:

C:\YourPath\

with your actual compiled DLL location.

11. Build a controlled test model

Don't test this first on a large production project.

Create a very small Revit test model.

Cable Tray

Create:

Cable Tray

Length = 6000 mm

Width  = 300 mm

Height = 100 mm

Horizontal.

Conduit

Create:

Conduit

Diameter = 50 mm

and make it physically cross the cable tray.

Something like:

CONDUIT

/

/

──────────────/──────────────

CABLE TRAY

Then run:

Cable Tray Clash Resolver

You should get:

Original:

──────────────────────────

/

/

/

After:

─────────╲

╲

╲────────────

╲

╲────────

with the new tray segments displayed pink.

12. One important point about the pink route

The pink color is only a test visualization.

The code uses Revit's per-element graphic override:

view.SetElementOverrides(...)

with:

SetProjectionLineColor(...)

Revit 2024 supports SetElementOverrides for view-specific element graphics. (Revit API Docs)

So the pink route helps us immediately distinguish:

BLACK = original

GREEN = existing conduit

PINK = automation result

This is extremely useful during development.

13. What I deliberately did NOT implement yet

There is a very important engineering reason.

Don't let us jump from this:

Cable Tray + Conduit

directly to:

Delete old tray

Create 6 trays

Create 4 elbows

Reconnect systems

We first need to prove the geometry.

Our development sequence should be:

V0.1

Automatic detection

↓

Automatic route calculation

↓

Create pink test geometry

Then:

V0.2

Multiple clashes

↓

Combine clash intervals

↓

One route per tray

Then:

V0.3

Clearance validation

↓

Recheck generated route

Then:

V0.4

Fitting generation

↓

Connector connection

Then:

V0.5

Split/remove original tray

↓

Reconnect original endpoints

Then:

V1.0

Production-ready automation

14. The next major problem we will solve

The current algorithm says:

“If a clash exists, go above the conduit.”

But the production algorithm needs to say:

“Find the best valid route around all obstacles.”

For example:

┌──────────────┐

/                \

────────────────┘                  └───────────────

CONDUIT

versus:

────────────────┐                  ┌───────────────

\                  /

└────────────────┘

The engine should evaluate:

ROUTE A

Above

Length = 5.2 m

Rise = 250 mm

Fittings = 4

Clearance = PASS

ROUTE B

Below

Length = 4.8 m

Rise = 180 mm

Fittings = 4

Clearance = FAIL

ROUTE C

Side

Length = 6.1 m

Fittings = 6

Clearance = PASS

Then:

ROUTE SOLVER

│

▼

┌──────────────┐

│ Score routes │

└──────┬───────┘

│

▼

BEST ROUTE

That is where this becomes a genuine Computational Design / BIM Automation solution.

15. One architectural correction I recommend now

Even though this is one .cs file for testing, do not keep the final project this way.

Once V0.1 works, we'll split it into:

CableTrayClashResolver

│

├── Commands

│   └── ResolveCableTrayClashesCommand.cs

│

├── Detection

│   ├── ElementCollector.cs

│   ├── BoundingBoxClashDetector.cs

│   └── SolidClashDetector.cs

│

├── Geometry

│   ├── GeometryExtractor.cs

│   ├── CoordinateSystem.cs

│   ├── ClashRegion.cs

│   └── VectorUtilities.cs

│

├── Routing

│   ├── RouteSolver.cs

│   ├── RouteCandidate.cs

│   ├── VerticalBypassSolver.cs

│   └── RouteScorer.cs

│

├── Revit

│   ├── CableTrayCreator.cs

│   ├── FittingCreator.cs

│   └── ConnectorService.cs

│

├── Validation

│   ├── ClearanceValidator.cs

│   ├── ClashValidator.cs

│   └── ConnectivityValidator.cs

│

└── Models

├── ClashInfo.cs

├── RouteInfo.cs

└── RoutingSettings.cs

That's the architecture I would take into a real BIM production environment.

The immediate target

Don't modify the code yet.

First, get this V0.1 into Revit 2024 and test it on a controlled model containing:

1 Cable Tray

×

1 Conduit

We want to verify these four things in order:

① It automatically finds both elements.

② It correctly reports the physical clash.

③ It creates the pink bypass.

④ The pink bypass has the correct width, height and elevation.

Once those four work, the next step is not UI — it is the most important part of the whole project:

Build the generalized 3D route solver so the pink path is calculated from the actual clash geometry rather than our current V0.1 “go above” rule.

That is where we'll start turning this prototype into the complete automatic Cable Tray Clash Resolution Engine.

Yes — this new image exposes two issues that we need to fix before going further:

- Orientation: the reroute cannot assume global X/Y. The tray can run in any horizontal direction, and the conduit can cross it at any angle.

- Fittings: the pink solution should be a real Cable Tray route with 45° transitions/bends, not merely several independent straight solids.

I would therefore change our development strategy from the previous V0.1 "go above" implementation to a local-coordinate + 45° route solver.

1. What your latest image tells me

The important geometry is this:

EXISTING CABLE TRAY

╲

╲

╲

╲

╲

╲

╲

────────────────

CONDUIT

The red/green highlighted areas show that this is happening inside a larger, rotated tray network.

So this is exactly where a global-coordinate implementation will fail.

We must stop thinking:

Z = Z + offset

X = clash.X

Y = clash.Y

and instead think:

Cable Tray Local Coordinate System

Z

↑

│

│

└──────────────→ X

Tray Direction

The entire calculation happens in this local system.

2. New architecture

Our routing pipeline should now become:

ACTIVE DOCUMENT

│

▼

Collect Cable Trays

│

▼

Collect Conduits

│

▼

Clash Detection

│

▼

Identify Tray Axis

│

▼

Build Tray Local Coordinate System

│

▼

Transform Clash

│

▼

Calculate Clearance

│

▼

Generate 45° Route

│

▼

Validate Route Geometry

│

▼

Create Cable Tray Segments

│

▼

Connect Segments

│

▼

Revit creates/uses

appropriate fitting

│

▼

Recheck Clash

This is much closer to the final architecture.

3. The key concept: Tray Local Coordinate System

Suppose the tray runs like this:

P1

/

/

/

/

P0

Its direction is:

XYZ trayDirection =

(P1 - P0).Normalize();

We then define:

XYZ X = trayDirection;

XYZ Z = XYZ.BasisZ;

XYZ Y = Z.CrossProduct(X).Normalize();

Therefore:

X = along tray

Y = across tray

Z = vertical

This means that regardless of whether the tray runs:

0°

15°

30°

45°

73°

127°

the algorithm still sees:

X = along tray

That's the major correction.

4. We should also calculate the conduit direction

Similarly:

LocationCurve conduitLocation =

conduit.Location as LocationCurve;

Line conduitLine =

conduitLocation.Curve as Line;

XYZ conduitDirection =

(conduitLine.GetEndPoint(1)

- conduitLine.GetEndPoint(0))

.Normalize();

Then calculate the relationship:

double angle =

trayDirection.AngleTo(conduitDirection);

This tells us whether we're dealing with:

Parallel

↓

Oblique

↓

Near perpendicular

We should not assume the conduit is perpendicular to the tray.

5. The 45° requirement changes the route mathematics

This is the most important change.

Previously we generated:

P0 ───── P1

\

P2────────P3

\

P4────P5

But now we want:

P0 ───────────── P1

\

\

P2────────────P3

\

\

P4 ─────────── P5

where:

P1 → P2 = 45°

P3 → P4 = 45°

So the vertical rise and horizontal transition are not independent.

For a 45° transition:

Vertical Rise = Horizontal Run

Therefore:

transitionDistance = verticalRise;

This gives us a very clean first-stage solver.

6. Example

Suppose:

Tray elevation = 3000 mm

Tray height    = 100 mm

Conduit top     = 3200 mm

Clearance       = 50 mm

Required tray center elevation:

3200 + 50 + 50

= 3300 mm

Therefore:

Rise = 3300 - 3000

= 300 mm

For a 45° transition:

Horizontal transition = 300 mm

So:

┌───────────────┐

/                 \

/                   \

──────────────┘                     └──────────────

300 mm              300 mm

↑

│ 300 mm

│

That's exactly the type of geometry we should generate.

7. But there is an important fitting issue

I want to correct something from my previous answer.

We should not assume that simply calling Connector.ConnectTo() will always create the exact 45° Cable Tray fitting we want.

Autodesk documents that Connector.ConnectTo() can create a fitting when necessary, but the connectors must be compatible and the resulting connection must be valid. (Autodesk Help)

For Cable Tray, Revit's own behavior is also fitting-aware: when cable tray is drawn with fittings, Revit automatically adds bends as needed. (Autodesk Help)

Therefore our initial strategy should be:

Strategy A — preferred

Create the route segments with geometry that naturally represents the 45° transitions, then use compatible Cable Tray connectors.

Strategy B

If Revit's automatic fitting resolution does not produce the desired 45° fitting, explicitly identify/create the appropriate Cable Tray fitting.

We will test A first.

8. Very important: Cable Tray Type

Your Cable Tray Type has an IsWithFitting property and a bend multiplier. Autodesk exposes these on CableTrayType. (Autodesk Help)

So before creating the route we should inspect:

CableTrayType trayType =

doc.GetElement(

originalTray.GetTypeId())

as CableTrayType;

Then:

bool withFitting =

trayType.IsWithFitting;

This is important.

If the original tray type is:

Cable Tray Type

With Fittings = YES

we should preserve that behavior.

If it is:

With Fittings = NO

we shouldn't blindly create fitting-dependent geometry.

9. The first 45° solver

Let's replace the previous TryBuildVerticalBypass() with this conceptual solver.

private static bool TryBuild45DegreeBypass(

ClashInfo clash,

out List<XYZ> points)

{

points = new List<XYZ>();

LocationCurve trayLocation =

clash.Tray.Location as LocationCurve;

if (trayLocation == null)

return false;

if (!(trayLocation.Curve is Line))

return false;

Line trayLine =

trayLocation.Curve as Line;

XYZ start =

trayLine.GetEndPoint(0);

XYZ end =

trayLine.GetEndPoint(1);

// --------------------------------------------------

// LOCAL TRAY AXIS

// --------------------------------------------------

XYZ X =

(end - start).Normalize();

XYZ Z =

XYZ.BasisZ;

XYZ Y =

Z.CrossProduct(X).Normalize();

// --------------------------------------------------

// TRAY DIMENSIONS

// --------------------------------------------------

double trayHeight =

GetParameterValue(

clash.Tray,

BuiltInParameter

.RBS_CABLETRAY_HEIGHT_PARAM);

// --------------------------------------------------

// CONDUIT BOUNDING BOX

// --------------------------------------------------

BoundingBoxXYZ conduitBox =

clash.Conduit.get_BoundingBox(null);

if (conduitBox == null)

return false;

// --------------------------------------------------

// REQUIRED ELEVATION

// --------------------------------------------------

double clearance =

50.0 / 304.8;

double minimumRise =

100.0 / 304.8;

double currentZ =

start.Z;

double targetZ =

conduitBox.Max.Z

+ clearance

+ trayHeight / 2.0;

targetZ =

Math.Max(

targetZ,

currentZ + minimumRise);

double rise =

targetZ - currentZ;

// --------------------------------------------------

// 45 DEGREE TRANSITION

//

// Horizontal run = vertical rise

// --------------------------------------------------

double transition =

rise;

// --------------------------------------------------

// FIND CLASH STATIONS

// --------------------------------------------------

double minStation;

double maxStation;

if (!TryGetStationRange(

clash.Intersection,

start,

X,

out minStation,

out maxStation))

{

return false;

}

// --------------------------------------------------

// ENTRY / EXIT

// --------------------------------------------------

double entry =

minStation - transition;

double exit =

maxStation + transition;

double trayLength =

start.DistanceTo(end);

if (entry <= 0)

return false;

if (exit >= trayLength)

return false;

// --------------------------------------------------

// POINTS

// --------------------------------------------------

XYZ p0 =

start;

XYZ p1 =

start + X * entry;

XYZ p2 =

p1 + X * transition

+ Z * rise;

XYZ p3 =

start + X * (maxStation);

// Better formulation:

// elevated run starts after the first 45°.

XYZ elevatedStart =

start + X * minStation

+ Z * rise;

XYZ elevatedEnd =

start + X * maxStation

+ Z * rise;

XYZ p4 =

elevatedEnd;

XYZ p5 =

start + X * exit;

XYZ p6 =

end;

AddPoint(points, p0);

AddPoint(points, p1);

AddPoint(points, elevatedStart);

AddPoint(points, elevatedEnd);

AddPoint(points, p5);

AddPoint(points, p6);

return points.Count >= 2;

}

However, I would not yet merge this into our main project, because there is an important geometric correction we need to make first.

10. Correct 45° route geometry

The route should actually be defined from stations, not by arbitrary points.

Let's define:

S0 = tray start

S1 = beginning of rise

S2 = end of rise

S3 = beginning of drop

S4 = end of drop

S5 = tray end

Then:

S2────────────────S3

/                    \

/                      \

S0──────────────S1                      S4──────────────S5

For 45°:

S2.X - S1.X = S2.Z - S1.Z

and:

S4.X - S3.X = S3.Z - S4.Z

This is much cleaner.

11. The algorithm should therefore become

CLASH INTERVAL

[SCLASH1,SCLASH2]

│

▼

┌─────────────────────┐

│ Add clearance       │

└──────────┬──────────┘

│

▼

Determine target Z

│

▼

Calculate rise

│

▼

45° transition distance

= vertical rise

│

▼

┌─────────────┴─────────────┐

▼                           ▼

45° UP                      45° DOWN

│                           │

└─────────────┬─────────────┘

▼

BEST ROUTE

For Stage 1, we will use:

45° UP ONLY

Then Stage 2 can introduce:

45° ABOVE

45° BELOW

and choose the valid route.

12. Orientation must also affect the clash station

This is where your latest model is especially useful.

We should calculate the clash location using:

double station =

(point - trayStart)

.DotProduct(trayDirection);

This is powerful.

It means:

Global X/Y/Z

doesn't matter.

For example:

Tray A

X direction = (1,0,0)

Tray B

X direction = (0.7,0.7,0)

Tray C

X direction = (-0.4,0.9,0)

The same algorithm handles all three.

13. We also need a local 2D representation

This is where I want to take the computational geometry one step further.

For every clash, convert the geometry into:

Local Z

↑

│

│

│

└────────────→ Local X

Tray direction

Then the conduit becomes a line/rectangle in this coordinate system.

We can therefore calculate:

Clash start X

Clash end X

Obstacle top Z

Obstacle bottom Z

Obstacle width

This makes the route solver independent of the Revit model orientation.

14. Fitting strategy

For your initial stage, I recommend:

Standard 45° transition

Cable Tray

──────────────╲

╲

╲

╲──────────────

and:

──────────────╲

╲

╲

╲──────────────

We will target:

45° up

45° down

with:

elevated straight segment

between them.

So the conceptual route is:

Existing tray

───────────────┐

╲

╲ 45°

╲

└──────────────────┐

╲

╲ 45°

└──────────────

This is much more like a real MEP routing solution.

15. How Revit fitting behavior helps us

Revit's Cable Tray tool itself automatically adds bends when drawing a run with fittings enabled. (Autodesk Help)

And Autodesk documents that Cable Tray and its fittings use physical connectors; connectors are automatically present on MEPCurve elements when created. (Autodesk Help)

Therefore after creating:

Segment A

Segment B

Segment C

we can inspect:

ConnectorManager

.Connectors

and connect compatible endpoints.

Conceptually:

Connector c1 = GetEndConnector(segmentA);

Connector c2 = GetEndConnector(segmentB);

c1.ConnectTo(c2);

ConnectTo() may create a fitting when necessary, provided the connection is valid. (Autodesk Help)

16. But I want one safety mechanism

After every connection:

Create segment

↓

Connect

↓

Regenerate

↓

Inspect

If Revit cannot create the expected fitting:

↓

Connection failure

↓

Rollback

We don't want:

Segment A

Segment B

Segment C

+

orphan geometry

in the production model.

17. The new V0.2 workflow

I recommend we now implement:

V0.2

AUTOMATIC

│

├── Collect all Cable Trays

│

├── Collect all Conduits

│

├── Detect clashes

│

├── Determine tray orientation

│

├── Determine conduit orientation

│

├── Calculate local coordinate system

│

├── Calculate clash interval

│

├── Calculate clearance

│

├── Calculate 45° rise

│

├── Generate route

│

├── Create tray segments

│

├── Connect segments

│

├── Check fitting result

│

└── Recheck clash

18. One thing I noticed in your screenshot that matters a lot

You have multiple cable tray paths and existing fittings in the model.

That means our eventual collector cannot simply treat every CableTray as an independent route.

We need to distinguish:

Straight CableTray

CableTray Fitting

CableTray network

The automatic collector should eventually build:

Cable Tray Network

│

├── Segment A

├── Segment B

├── Segment C

├── Fitting 1

├── Fitting 2

└── Segment D

Then if Segment B clashes:

Network

│

▼

Determine affected segment

│

▼

Preserve network endpoints

│

▼

Reroute only affected portion

This will be essential before we start deleting/replacing the original geometry.

19. Another major correction: don't use bounding-box Z blindly

Our previous prototype uses:

conduitBox.Max.Z

That is acceptable for V0.1 testing, but not for the final solver.

Because if the conduit is rotated, the axis-aligned bounding box may be much larger than the actual conduit.

Instead, eventually we'll calculate:

Conduit axis

+

Conduit radius

+

Clearance

to produce a proper clearance envelope.

For a conduit:

Conduit centerline

│

│

( )

(   )

( )

│

we calculate the required envelope from:

radius

+

clearance

+

tray half-height/width

rather than trusting the global bounding box.

That will make the solution considerably more reliable.

20. Our final routing mathematics

Eventually, the solver will work approximately like this:

Conduit axis

/

/

/

───────────────/──────────────

Cable Tray axis

Transform into tray-local coordinates:

Z

↑

│

╭───────┴───────╮

│   Forbidden   │

│     region    │

╰───────────────╯

│

└────────────────→ X

Then find:

X_entry

X_exit

Z_required

and construct:

┌──────────────────┐

/                    \

/                      \

──────────┘                        └──────────

with:

45° up

horizontal

45° down

Then transform back to world coordinates.

That is the right computational-geometry foundation.

21. What I recommend we implement next

Rather than modifying ten things simultaneously, I want our next build to have exactly these four deliverables:

V0.2-A — Orientation Engine

TrayCoordinateSystem

Input:

CableTray

Output:

Origin

X

Y

Z

V0.2-B — Clash Station Engine

Input:

CableTray

Conduit

Intersection Solid

Output:

ClashStart

ClashEnd

in tray-local coordinates.

V0.2-C — 45° Route Solver

Input:

ClashStart

ClashEnd

Conduit clearance

Tray height

Output:

P0

P1

P2

P3

P4

P5

with guaranteed:

P1→P2 = 45°

P3→P4 = 45°

V0.2-D — Fitting/Connection Test

Create:

Segment 1

Segment 2

Segment 3

then attempt:

Connector → Connector

and verify that Revit produces the expected Cable Tray fitting.

Autodesk confirms that Cable Tray connectors exist automatically on newly created MEPCurve elements and that Connector.ConnectTo() can establish connections and, when needed, create fittings. (Autodesk Help)

The target result

Your current screenshot:

BLACK = existing tray

GREEN = conduit

should eventually become:

BLACK

──────────────────────────╲

╲

╲

───────────────

╲

╲

───────────────

GREEN

╲

╲

╲

PINK = generated

And critically, the pink route should be generated from the actual model geometry, regardless of whether the original tray is rotated 0°, 30°, 45°, 90° or some arbitrary angle.

For Stage 1, we'll standardize on 45° vertical transitions, then later add above/below selection, multiple obstacles, fitting-radius constraints, network preservation, and optimization.

This is the point where I would consider the project to have moved from a simple Revit macro into a genuine computational MEP routing engine.

Absolutely. This is the correct next improvement. Looking at your latest image, I would now separate the problem into two parts:

- Route geometry — calculate the correct 45° rise/fall.

- Fitting geometry — make Revit create the actual Cable Tray fittings at those direction changes, instead of leaving sharp/open connections.

The important correction is: we should not draw a polyline and assume that Revit will visually produce the desired fitting. We need to deliberately create connected Cable Tray segments and validate the resulting connectors/fittings.

Autodesk confirms that newly created MEPCurve elements have physical connectors and that Connector.ConnectTo() can create a fitting when required. (Autodesk Help) Revit also exposes the Cable Tray Type's default elbow and IsWithFitting properties. (Autodesk Help)

1. The geometry we want

Your current target is essentially:

┌───────────────┐

/                 \

/                   \

─────────────────┘                     └─────────────────

↑                     ↑

45° fitting           45° fitting

But in 3D:

Elevated tray

─────────────────────

╱        ╲

╱          ╲

╱            ╲

─────────────────╯              ╰─────────────────

existing tray                         existing tray

So we need:

Segment A → 45° UP fitting → Segment B → 45° DOWN fitting → Segment C

Not:

Segment A → random diagonal → Segment B

2. Very important: your R annotation

I noticed that you marked R at the two transition locations.

That is important.

The actual fitting has a bend radius, and we should not simply use:

horizontalRun = verticalRise;

and assume that is the complete fitting geometry.

For the initial implementation, however, I recommend:

Use the project's Cable Tray Type fitting definition and let Revit resolve the physical fitting through connectors.

The route solver controls the 45° direction.

The Revit fitting controls the physical bend radius.

That is much safer than trying to manually model the fitting ourselves.

3. First thing to add: fitting diagnostics

Before changing the route algorithm, add this utility.

private static List<Connector> GetConnectors(Element element)

{

var result = new List<Connector>();

MEPCurve mepCurve = element as MEPCurve;

if (mepCurve == null)

return result;

ConnectorSet connectors =

mepCurve.ConnectorManager.Connectors;

foreach (Connector connector in connectors)

{

result.Add(connector);

}

return result;

}

Then:

private static Connector GetNearestConnector(

Element element,

XYZ point)

{

Connector nearest = null;

double minDistance = double.MaxValue;

foreach (Connector connector in GetConnectors(element))

{

double distance =

connector.Origin.DistanceTo(point);

if (distance < minDistance)

{

minDistance = distance;

nearest = connector;

}

}

return nearest;

}

This gives us direct control over the Cable Tray connection points.

4. Add connector debugging

This will be extremely useful while developing.

private static void DumpConnectors(

Element element)

{

foreach (Connector c in GetConnectors(element))

{

TaskDialog.Show(

"Cable Tray Connector",

$"Element: {element.Id}\n" +

$"Origin: {c.Origin}\n" +

$"Direction: {c.CoordinateSystem.BasisZ}\n" +

$"Domain: {c.Domain}\n" +

$"Connected: {c.IsConnected}");

}

}

Later we'll replace TaskDialog with a proper logger.

For example:

Tray 123456

Connector 1

Origin = (100,200,300)

Direction = (1,0,0)

Connector 2

Origin = (500,200,300)

Direction = (-1,0,0)

This lets us understand exactly what Revit thinks the tray endpoints are.

5. Create the Cable Tray segments

For the test, we'll use the original tray's:

- Type

- Width

- Height

- Level

and only change the geometry.

private static CableTray CreateTraySegment(

Document doc,

CableTray sourceTray,

XYZ start,

XYZ end)

{

return CableTray.Create(

doc,

sourceTray.GetTypeId(),

start,

end,

sourceTray.LevelId);

}

This is important because we don't want to accidentally create a different Cable Tray type.

6. The first fitting test

Now we create two non-collinear segments.

Segment B

/

/

/

────

Segment A

Then:

private static bool TryConnect(

Connector connector1,

Connector connector2)

{

if (connector1 == null ||

connector2 == null)

return false;

try

{

connector1.ConnectTo(connector2);

return true;

}

catch (Exception ex)

{

System.Diagnostics.Debug.WriteLine(

"Connection failed: " + ex.Message);

return false;

}

}

This is where Revit gets the opportunity to insert the fitting.

Autodesk specifically documents that ConnectTo() may create a new fitting when required. (Autodesk Help)

7. But don't blindly connect nearest connectors

This is an important senior-level correction.

We shouldn't do:

GetNearestConnector(A)

GetNearestConnector(B)

ConnectTo()

without checking direction.

We need:

Position

+

Direction

+

Domain

+

Shape

+

Size

So create a validator.

private static bool AreConnectorsCompatible(

Connector a,

Connector b,

double tolerance)

{

if (a == null || b == null)

return false;

if (a.Domain != b.Domain)

return false;

double distance =

a.Origin.DistanceTo(b.Origin);

if (distance > tolerance)

return false;

return true;

}

For the first test:

double tolerance =

5.0 / 304.8; // 5 mm

8. Now the 45° route builder

This is the part I recommend replacing our previous route creation logic with.

private class RoutePoints

{

public XYZ Start;

public XYZ UpStart;

public XYZ UpEnd;

public XYZ DownStart;

public XYZ DownEnd;

public XYZ End;

}

And:

private static RoutePoints Build45DegreeRoute(

XYZ start,

XYZ end,

XYZ trayDirection,

double targetElevation,

double clashStart,

double clashEnd)

{

XYZ X =

trayDirection.Normalize();

XYZ Z =

XYZ.BasisZ;

double rise =

targetElevation - start.Z;

if (rise <= 0)

throw new InvalidOperationException(

"Target elevation must be above original tray.");

// 45 degrees:

// horizontal distance = vertical distance

double transition =

rise;

double upStartStation =

clashStart - transition;

double upEndStation =

clashStart;

double downStartStation =

clashEnd;

double downEndStation =

clashEnd + transition;

RoutePoints route =

new RoutePoints();

route.Start =

start;

route.UpStart =

start +

X * upStartStation;

route.UpEnd =

start +

X * upEndStation +

Z * rise;

route.DownStart =

start +

X * downStartStation +

Z * rise;

route.DownEnd =

start +

X * downEndStation;

route.End =

end;

return route;

}

The resulting geometry is:

UpEnd

●────────────●

╱                ╲

╱                    ╲

╱                        ╲

Start ●──────────●                          ●──────────● End

UpStart                  DownEnd

9. But we need one correction in that code

The above assumes:

start.Z == end.Z

and that the route is aligned with the tray's local X direction.

That's fine for the first controlled test.

But your model contains rotated trays.

So our production version should instead calculate stations using:

double station =

(point - trayStart)

.DotProduct(trayDirection);

rather than global X coordinates.

That keeps the solution orientation-independent.

10. Create the three Cable Tray segments

Now:

private static List<CableTray> Create45DegreeRoute(

Document doc,

CableTray sourceTray,

RoutePoints p)

{

var trays =

new List<CableTray>();

CableTray segment1 =

CreateTraySegment(

doc,

sourceTray,

p.Start,

p.UpStart);

CableTray segment2 =

CreateTraySegment(

doc,

sourceTray,

p.UpEnd,

p.DownStart);

CableTray segment3 =

CreateTraySegment(

doc,

sourceTray,

p.DownEnd,

p.End);

trays.Add(segment1);

trays.Add(segment2);

trays.Add(segment3);

return trays;

}

Now we have:

Segment 1

Segment 2

────────────

Segment 3

But there are still gaps between:

Segment1 → Segment2

Segment2 → Segment3

That's intentional.

Those gaps are where Revit should resolve the fittings.

11. Connect the segments

Now:

private static void ConnectTraySegments(

Document doc,

CableTray first,

CableTray second,

CableTray third)

{

Connector firstEnd =

GetConnectorClosestTo(

first,

second.Location);

Connector secondStart =

GetConnectorClosestTo(

second,

first.Location);

if (firstEnd != null &&

secondStart != null)

{

firstEnd.ConnectTo(secondStart);

}

Connector secondEnd =

GetConnectorClosestTo(

second,

third.Location);

Connector thirdStart =

GetConnectorClosestTo(

third,

second.Location);

if (secondEnd != null &&

thirdStart != null)

{

secondEnd.ConnectTo(thirdStart);

}

doc.Regenerate();

}

However, I don't like the Location approach here.

Let's make the connector selection geometrically deterministic.

12. Better connector selection

private static Connector GetConnectorClosestTo(

Element element,

XYZ target)

{

Connector best = null;

double bestDistance =

double.MaxValue;

MEPCurve curve =

element as MEPCurve;

if (curve == null)

return null;

foreach (Connector connector

in curve.ConnectorManager.Connectors)

{

double distance =

connector.Origin.DistanceTo(target);

if (distance < bestDistance)

{

bestDistance = distance;

best = connector;

}

}

return best;

}

Then:

Connector c1 =

GetConnectorClosestTo(

segment1,

p.UpStart);

Connector c2 =

GetConnectorClosestTo(

segment2,

p.UpEnd);

But there's another subtle issue.

13. We actually want the connector at the fitting location

This is the important part.

Our route should look like:

Segment 2

─────────────────

↑

/

/

/

Segment 1 ──────

The connector positions must be compatible with the fitting's geometry.

Therefore, the fitting radius must eventually be included in our route solver.

For V0.2, however, we can let Revit determine whether the selected Cable Tray Type can resolve the connection.

14. Inspect the Cable Tray Type before routing

Add:

private static void InspectTrayType(

CableTray tray)

{

CableTrayType type =

tray.Document.GetElement(

tray.GetTypeId())

as CableTrayType;

if (type == null)

return;

System.Diagnostics.Debug.WriteLine(

$"Cable Tray Type: {type.Name}");

System.Diagnostics.Debug.WriteLine(

$"With Fitting: {type.IsWithFitting}");

System.Diagnostics.Debug.WriteLine(

$"Bend Multiplier: {type.BendMultiplier}");

if (type.Elbow != null)

{

System.Diagnostics.Debug.WriteLine(

$"Elbow Type: {type.Elbow.Id}");

}

}

The API exposes Elbow, BendMultiplier, and IsWithFitting on CableTrayType. (Autodesk Help)

This is something I want us to capture in our debug report.

15. We should also inspect the generated fitting

After:

connector1.ConnectTo(connector2);

run:

doc.Regenerate();

Then collect Cable Tray fittings.

private static List<CableTrayFitting>

GetCableTrayFittings(

Document doc)

{

return new FilteredElementCollector(doc)

.OfClass(typeof(CableTrayFitting))

.Cast<CableTrayFitting>()

.ToList();

}

Then we can identify fittings created during the transaction.

16. Better: track fittings before and after

HashSet<ElementId> GetFittingIds(

Document doc)

{

return new FilteredElementCollector(doc)

.OfClass(typeof(CableTrayFitting))

.ToElementIds()

.ToHashSet();

}

Before route:

HashSet<ElementId> before =

GetFittingIds(doc);

Create/connect route.

Then:

doc.Regenerate();

HashSet<ElementId> after =

GetFittingIds(doc);

Find:

var newFittings =

after

.Except(before)

.ToList();

Now we know exactly what Revit generated.

17. This is extremely important for our debugging

Our report should say something like:

========================================

CABLE TRAY REROUTE REPORT

========================================

Original Tray : 123456

Tray Type     : Ladder 300x100

Original Direction:

X = 0.707

Y = 0.707

Z = 0.000

Clash:

Start Station = 2450 mm

End Station   = 2850 mm

Target Elevation:

+300 mm

Route:

45° UP

Horizontal

45° DOWN

Segments:

A = 2150 mm

B = 400 mm

C = 2150 mm

Fittings Created:

Fitting 781234

Fitting 781235

Fitting 1:

Angle = 45°

Fitting 2:

Angle = 45°

Final Clash:

FALSE

========================================

This is the level of diagnostic information I want in the final automation.

18. One thing I do NOT want us to do

I don't recommend manually placing a generic family and trying to make it look like a Cable Tray fitting.

For example, don't do:

doc.Create.NewFamilyInstance(...)

for a generic elbow.

That would produce something visually similar but not necessarily a proper Cable Tray network.

Revit has an explicit Cable Tray Fitting category/command, and Cable Tray fittings are intended to be connected to the tray network. (Autodesk Help)

Our target is:

CableTray

↓

CableTrayFitting

↓

CableTray

not:

CableTray

↓

Generic Model Family

↓

CableTray

19. Another important point: 45° vs radius

This is where we will make the next refinement after this test.

Suppose:

R = 150 mm

and we want:

45°

The fitting is not simply a mathematical point.

It occupies physical space.

So eventually our solver must understand:

┌──────────────

/

/

/

R

and calculate the tangent points.

For an ideal circular bend, the tangent offset relationship is:

T = R × tan(θ / 2)

For:

θ = 45°

that becomes:

T ≈ 0.4142R

But we should not hard-code that as the Revit fitting geometry yet, because the actual loaded Cable Tray fitting family/type is authoritative.

Instead:

Stage 1

Route solver

↓

45° direction

↓

Revit connectors

↓

Revit fitting

Stage 2

Route solver

↓

Read actual fitting/bend properties

↓

Calculate tangent points

↓

45° + actual R

↓

Create route

That's the robust approach.

20. I would structure the code now like this

Our project is starting to become a real routing engine, so I would stop putting everything in one Execute() method.

CableTrayAutoRouter

│

├── Collection

│   ├── CableTrayCollector

│   └── ConduitCollector

│

├── Clash

│   ├── ClashDetector

│   └── ClashInfo

│

├── Geometry

│   ├── TrayCoordinateSystem

│   ├── StationCalculator

│   └── GeometryUtils

│

├── Routing

│   ├── RouteSolver

│   ├── RoutePoints

│   └── FortyFiveDegreeSolver

│

├── Revit

│   ├── CableTrayCreator

│   ├── ConnectorUtils

│   └── FittingManager

│

└── Validation

├── ClashValidator

├── FittingValidator

└── RouteValidator

That architecture will save us a lot of pain when we add multiple obstacles later.

21. Our immediate test case

For the model shown in your screenshot, I want the first test to be deliberately simple:

CONDUIT

│

│

●

Original Cable Tray

──────────────────────────────

Automation:

┌───────────────────────┐

/                         \

/                           \

───────────┘                             └───────────

And the program should produce:

1 original clash detected

2 45° transition points calculated

3 Cable Tray segments created

4 first connector connected

5 first fitting generated

6 second connector connected

7 second fitting generated

8 final route regenerated

9 clash check executed

10 result = PASS

22. One more correction to our previous implementation

I would not yet delete the original tray.

For the next test:

Original Tray

+

Generated Tray

+

Generated Fittings

should all remain temporarily.

We can color/mark the generated elements.

Once the geometry is proven:

Original Tray

↓

Split / remove affected section

↓

Generated route

will become the production operation.

This gives us rollback safety while developing.

23. Recommended V0.2 test sequence

Test 1 — horizontal tray

Tray = 0°

Conduit = perpendicular

Expected:

45° UP

────────

45° DOWN

Test 2 — rotated tray

Tray = 30°

Conduit = perpendicular

Expected: identical local solution, rotated 30° in world space.

Test 3 — rotated conduit

Tray = 30°

Conduit = 60°

Expected: clash station still correctly calculated.

Test 4 — different tray width

100 mm

150 mm

300 mm

Expected: required clearance changes accordingly.

Test 5 — different bend radius

R = project default

Expected: Revit fitting selection remains valid.

The key implementation decision

For this next build, I recommend we don't try to manually construct the Cable Tray fitting family.

We will use:

Geometry solver → Cable Tray segments → physical connectors → ConnectTo() → Revit Cable Tray fitting → fitting validation.

That is much closer to how Revit itself expects Cable Tray networks to be constructed. Autodesk explicitly states that Cable Tray can be drawn with fittings and that ConnectTo() can create a fitting when necessary. (Autodesk Help)

And your R-marked requirement becomes our next geometry layer: after we prove the 45° fitting connection works, we'll make the solver read/consider the actual fitting bend radius rather than treating the transition as an infinitely sharp mathematical corner.

So I would call the next milestone V0.2 – 45° Cable Tray Fitting Engine. Once that passes on your exact Revit model, we can move directly into the more difficult part: preserving the original tray network, cutting only the conflicted portion, inserting the new route, and automatically validating that the final model has zero cable-tray/conduit clashes.